Traverse Drive not working
PROBLEM--Traverse Drive not working in (manual) jog mode

Assuming 115 Volts AC to control panel and all other manual (jog) mode functions are working.

Possible Cause

Traverse Speed Pot (TSP) set to zero

Checkout Procedure

A. Set (TSP) to 35 on the control panel
[image: image1.emf]
Traverse works

Yes--end troubleshooting

No--go to step B. next
Possible Cause

Traverse Belt Clamp release lever released

Checkout Procedure

B. Insure release lever is in adjusted properly. See Adjustments section

of this manual.

[image: image2.emf]
Traverse works

Yes--end troublshooting

No--go to Step C. next

Possible Cause 
Circuit Breaker 32 (CB32) (3 amp) tripped

Checkout Procedure

C. Too heavy a grind causes grinding head traverse motor to overload and trip the circuit breaker. Reset (CB32)

[image: image3.emf]
Traverse works

Yes--end troublshooting

No--go to Step D. next
Possible Cause

Traverse Drive Control (TDC) do not have power
Checkout Procedure
D. Check for 115 Volts AC incoming to (TDC) (insure traverse right or left has been

pressed at least once)
[image: image4.emf]
[image: image5.jpg]



On (TDC) Term L1 to L2 for 115 Volts AC

Yes--Skip to Step H.

No--go to Step E. next
Possible Cause

Relay 8 (RT1) is bad

Checkout Procedure
E. Check for relay 8 (RT1) light on (insure traverse right or left has been pressed at least

once)

[image: image6.jpg]D::mw:m:mmmmmmmmm@mww

,A mm 00 1020304050607 B YoAOBO LODOEGF| | (e
7 L3 3 3 Y 0 0 0 0 0 0 0 0 - [ ey v

, mm @ .I“Cm@mlmlmﬁm@ .@ml “l_i inlulmlﬂl_ ©f _I QX ® R

..u
ﬁ

llllll

o
ol ol ol od o ad
I 5| 9| - J 4 2
e ul & N





Light is:

On-- go to Step F. next

Off-- Contact Factory
Checkout Procedure
F. Check relay 8 for continuity,

Insure relay 8 light is on
[image: image7.jpg]403000 o0HdoVoE080,08050p0F€

S SNEEEEEEEES

TIN




(RT1) Term 8+ to 8- read 115 Volts AC

Yes-- Replace Relay 8 (RT1)

No-- go to Step G. next
Possible Cause

Circuit Breaker 32 (CB32) bad [image: image8.emf]
Checkout Procedure
G. Check CB32 for voltage
[image: image9.jpg]



(CB32) from Line neutral,
[image: image10.jpg]



 Light Blue wire at the line filter wire # 02FTRBU, to wire #156CB32-BL at CB32 
[image: image11.jpg]156C832-BL

3|




measure 115 Volts AC:

No-- Replace CB32

Yes-- Verify continutiy of wires to RT1. Replace or repair bad wire(s)

Possible Cause
No DC Voltage from (TDC)

Traverse Drive Control
Checkout Procedure
H. Check for 90 Volts DC across (TDC) terminals A! to A2 this voltage drives the DC traverse motor.

NOTE: Traverse must be on and have (TSP) turned full CW to maximum voltage of 90 VDC

[image: image12.jpg]



Check (TDC) terminals A1 to A2 for 90 Volts DC

Yes--go to step I. next

No--go to Step J. next

Note:If voltage is less than 90 VDC verify pots on TDC. 
Possible Cause
Traverse Motor is bad

Checkout Procedure
I. Check traverse motor continuity

Remove wires from terminals A1 & A2  0 ohms across the black and white wires

Yes--go to Step J. next

No--go to Step N.
Possible Cause
Check Relays 2 and 3

Checkout Procedure
J. (RT1) Verify that relay 2 light comes on when Traverse Right is pressed, and that relay 3 light comes on when Traverse Left is pressed

[image: image13.jpg]D::mw:m:mmmmmmmmm@mww

,A mm 00 1020304050607 B YoAOBO LODOEGF| | (e
7 L3 3 3 Y 0 0 0 0 0 0 0 0 - [ ey v

, mm @ .I“Cm@mlmlmﬁm@ .@ml “l_i inlulmlﬂl_ ©f _I QX ® R

..u
ﬁ

llllll

o
ol ol ol od o ad
I 5| 9| - J 4 2
e ul & N





Lights come:On-- go to step K. next

                    Off-- Skip to step L.

Possible Cause
(TSP) (10K) is bad

Checkout Procedure
K. Check (TSP) for 10,000 ohms

Remove three wires from (TDC) [image: image14.emf]
red from term S2 

white from term S0 

black from inline connector (Wire 39)

[image: image15.jpg]



Check for 10,000 ohms red to white wires

Full CCW--0 ohms Full CW--10,000 ohms

Red to black wires

Full CCW--10,000 ohms Full CW--0 ohms

Yes--go to Step L. next

No--replace (TSP)
Possible Cause

Gap between flag and prox is incorrect.
Checkout Procedure

L. Gap between flag and Prox

should be 3/16" to 1/4" [4-6mm].

Prox light does not light when flag is

under prox.

If incorrect, adjust 
Traverse works—

Yes-- End troubleshooting

No-- go to step M. next

Possible Cause

Proximity switch is bad

M. From the Touch screen, Enter

"HELP" screen from main menu.

Follow instructions on screen to verify

traverse proximity switches are ok.

[image: image16.emf]
[image: image17.emf]
HELP SCREEN

This help screen will assist in verifying that the proximity switches are working

correctly. With this screen displayed, place a metal object in front of the

proximity switch in question. The light on the proximity switch should come

on and the symbol next to the prox on this screen should turn from grey to

red. If the symbol does not turn red, then the prox is not fuctioning properly

and more analysis is needed. If this is the case press the "Next" and follow

the instructions or consult the Assembly & Service Manual
Switch is

Good-- Replace (TDC)

Bad-- Replace switch
Traverse works—

Possible Cause
Worn motor brushes
Checkout Procedure
N. Inspect Motor Brushes
Remove the brushes one at a time and maintain orientation for reinsertion.

See if brush is worn short, 3/8" [10 mm] minimum length.

[image: image18.emf]
Yes--replace motor brushes

No--replace Traverse Drive Motor
